reactance is the opposition of a circuit element to a change in current or voltage, due to that element's inductance or capacitance.

In phasor analysis, reactance is used to compute amplitude and phase changes of sinusoidal alternating current going through a circuit element. It is denoted by the symbol {\displaystyle \scriptstyle {X}}. An ideal resistor has zero reactance, whereas ideal inductors and capacitors have zero resistance – that is, respond to current only by reactance. The magnitude of the reactance of an inductor rises in proportion to a rise in frequency, while the magnitude of the reactance of a capacitor decreases in proportion to a rise in frequency. As frequency goes up, inductive reactance goes up and capacitive reactance goes down.

An LC circuit, also called a resonant circuit, tank circuit, or tuned circuit, is an electric circuit consisting of an inductor, represented by the letter L, and a capacitor, represented by the letter C, connected together. The circuit can act as an electrical resonator, an electrical analogue of a tuning fork, storing energy oscillating at the circuit's resonant frequency.

LC circuits are used either for generating signals at a particular frequency, or picking out a signal at a particular frequency from a more complex signal; this function is called a bandpass filter. They are key components in many electronic devices, particularly radio equipment, used in circuits such as oscillators, filters, tuners and frequency mixers.

An LC circuit is an idealized model since it assumes there is no dissipation of energy due to resistance. Any practical implementation of an LC circuit will always include loss resulting from small but non-zero resistance within the components and connecting wires. The purpose of an LC circuit is usually to oscillate with minimal damping, so the resistance is made as low as possible.

The two-element LC circuit described above is the simplest type of inductor-capacitor network (or LC network). It is also referred to as a second order LC circuit to distinguish it from more complicated (higher order) LC networks with more inductors and capacitors. Such LC networks with more than two reactances may have more than one resonant frequency.